232/1

PHYSICS 	LANJET 2021

PAPER 1

MARKING SCHEME

- 16.21mm1 correct answer with correct units

Accept 1.621cm or 0.01621m

Magnetic force

Electrostatic force

Gravitational force

- Momentum is conserved momentum before = momentum after

72 ×9 = 216 ×41

u = 72 ×9216 1

= 3.0m/s1

- Roofing materials allows radiations to penetrate into the greenhouse1 but not out. Higher concentration of carbon dioxide inside the greenhouse helps to retain higher temperature by trapping/ insulating1 the heat.

- V.R = 31

- Increase in temperature increases1 the speed of sound.

- i)	Convection takes place in air upwards direct 	due to1 to density defect.

ii) 	Convection requires a 1 material medium 	but the space between the sun and the earth 	i.e. space of the atmosphere has no material 	medium

- From the equation of continuity

A1U1 = A2U2 1(flow rate is constant)

120 × 0.4 = 4 × U2

∴ U2 = 120 ×0.44 1

= 12 ms-11

- Work done on the mass

= force × distance

= 25 ×10 × 120

= 5000J. 1

Work done = power × time

= 200 × 301

= 6000J1

But  = work outputwork input ×100

= 50006000 × 100

= 83.3% 1

- H = MCθ

= 1501000 ×1000×4200 ×70-15 +

390 ×20×70-15

= 34650.000 + 429000

= 4636501

Energy dissipation E = pt

3000 ×t=4636501

t = 4636503000 = 154.55 sec1

- At balance

Sum of clockwise = sum of anti-clockwise moments

1801000×100× 40 = 30 ×X + (10×1.8) 1

1.8×40 = 30X +18

X = 1.8 ×40-1830 1

= 1.8N1

- To increase surface area of contact thus reducing pressure exerted on the road1

SECTION B

13. (a) (i) A-B   - stationary body

(ii) B – C to moves with increasing velocity

(iii) CD to deceasing velocity

50m

45m

b)

R = Ut

h = ½ gt2

45 = ½ x 10 x t

t2 = 45 = 9

5

t = 3less

14.a)	The pressure of a fixed mass of gas is	 	directly proportional to absolute temperature 	provided that volume is kept constant. 1

b)	When a gas is heated it expands 1 	increasing the number of collusions per unit 	area which in turn raise the pressure1

c)

i)	-	Temperature of water1

-	Pressure gauge reading1

ii)	-	Collect various values of pressure under 		different temperatures1

-	Plot a graph of pressure against 			temperature and study the values1

iii)	P α T →P=KT →K=PT

P1 – 5.0 x 105 Pa

P2 = ?

T1 = 273 + 37 = 300k

T2 = 75 + 273 = 348k

P1T1= P2T2

500000300= P2348 1

P2 = 500000 ×348300

= 580000

P2 = 5.8 x 105 Pa1

(ii)     R = ut

50 =  0 x 3

3         3

U =  50 = 16. 67 m/s

3

(iii)   V = u  + at

= gt

= 10 x 3

=30m/s

15.a ) i) 	In elastic collision – K.E and 			momentum of the objects are 			conserved1

Elastic collision – only momentum is 		conserved1

ii) 	Initial momentum = Final momentum

b) 	i)

ii)

16.a) i) 	ω= 2πf

= 2π × 10

= 20πrad s-1

= 62.83 rad-1

TA = M ω 2r – mg

= 1×62.832 ×0.5- 1×101

= 19 + 3.9 – 10

= 1963.9N1

ii) 	At the lowest point

Fc = T – Mg

Fe + Mg

= mr ω 2 + mg

= 1×0.5×62.832+ 1×101

= 1973.9 + 10

= 1983.9N1

b(a)	(i)	Tension in the spring supporting the object. ¹

(ii)	There is change in the direction of instantaneous velocity at various

points along the circular path. ¹

(b)	(i)	The spring balance reading increases. ¹

(ii)	The centripetal force

- The object moves tangentially to the circular path at that point where it cuts. ¹

a) 	17 (a) A floating object displaces its own weight of the fluid in which it float.

b (i)

Upthrust, u

(ii) 	U = mg + T

(ii)  	Viscous drag

B	(i) 	Upthrust = weight of solid

=  11.5 x 0.8 x 10

1000

= 0.092 N

(ii) Density of solid	= volume submerged

Density of liquid	         Total volume

0.8    = 8.5

e1	11.5

e1   = 1.0824g/cm3

Alternatively

e = m

v

=  0. 0.92 x 100

8.5

1mk

= 1.0824g/cm3

d) i)

| Load | 0.00 | 1.00 | 2.00 | 4.00 | 5.00 | 6.00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L | 10.00 | 11.50 | 13.50 | 16.00 | 18.00 | 24.00 |  |
| Weight (mg) E | 0.00 | 1.50 | 3.50 | 6.00 | 8.00 | 14.00 | 1 |

Tension (T)

ii) Suitable axes labelled 1

All points correct 1

Suitable line 1

iii) Springs constant K = F   1

e

Use students graph

Correct units 1

iv) Energy stored when the length is stretched by 16 cm

Area under the graph 1

Or E = ½ ke2

Use k from graph and e = 16 cm.

K must be correct.

Correct substitution 1

Answer correct unit 1

(iii) Upthrust  = weight of liquid displaced

= 11.5 x 1.0824   x 10 		                                           1000

=  0.1245 N

18.